ME5309 Aircraft Engines and Rocket Propulsion

Assignment 2: Ideal and Non-Ideal Cycle Analysis

Part (a)

Due to the engine is ideal, and with the altitude goes higher, the temperature will follow:

T=288.15-0.0065H (H<11000m)216.65K (11000≤H≤20100m)

And do it in the MATLAB, the coding will be attached behind.

Part (b)

For this case, at different altitude the maximum specific thrust happens in different height, so first of all is to fix the height and change the parameter πc to find out the best specific thrust performance at this altitude, and change the altitude to do the same over and over again. And the result is shown:

Part (c)

The turbojet is not ideal, so we need to do more calculations, and add them into the code.

Part (d)

In this part, ec  and et have a variance of ±2%. So for ec,et ∈[0.833,0.867].

- For ec∈[0.833,0.867]

- For et∈[0.833,0.867]

From two charts above, we can conclude that the specific thrust is more sensitive to ec.

Part (e)

Fix the height to 10km:

Appendix

Part(a)

clc;

clear;

clear all;

%known parameters

H_values = linspace(0,15000,1000);%%define the height

M_0_values = [0.85,2.1];

gamma = 1.4;

R = 287;

C_p = R * gamma/(gamma-1);

T_t4 = 1900;

pi_c = 15;

Qr = 43000000;

%need to be known

T_0_values = zeros(size(H_values));

a_0_values = zeros(size(H_values));

V_0_values = zeros(size(H_values));

t_r_values = zeros(size(H_values));

t_lambda_values = zeros(size(H_values));

pi_r_values = zeros(size(H_values));

t_c_values = zeros(size(H_values));

eff_c_values = zeros(size(H_values));

f_values = zeros(size(H_values));

t_t_values = zeros(size(H_values));

pi_t_values = zeros(size(H_values));

P_t9_9_values = zeros(size(H_values));

M_9_values = zeros(size(H_values));

T_9_T_0_values = zeros(size(H_values));

V_9_a_0_values = zeros(size(H_values));

thrust_values = zeros(length(M_0_values),length(H_values));

TSFC_values = zeros(length(M_0_values),length(H_values));

eff_p_values = zeros(length(M_0_values),length(H_values));

eff_th_values = zeros(length(M_0_values),length(H_values));

eff_o_values = zeros(length(M_0_values),length(H_values));

%definr the T_0

for i = 1:length(H_values)

H = H_values(i);

if H < 11000

T_0_values(i) = 288.15-0.0065*H;

else

T_0_values(i) = 216.65;

end

for j=1:length(M_0_values)

M_0 = M_0_values(j);

a_0_values(i) = sqrt(gamma * R * T_0_values(i));

V_0_values(i) = a_0_values(i) * M_0;

t_r_values(i) = 1+(gamma-1)/2 * M_0^2;

pi_r_values(i) = t_r_values(i)^(gamma/(gamma - 1));

t_lambda_values(i) = T_t4 / T_0_values(i);

t_c_values(i) = pi_c^((gamma-1)/gamma);

f_values(i) = (t_lambda_values(i)-t_r_values(i) * t_c_values(i)) ./ (Qr/(C_p * T_0_values(i))-t_lambda_values(i));

t_t_values(i) = 1-t_r_values(i)*(t_c_values(i)-1)/t_lambda_values(i);

pi_t_values(i) = t_t_values(i)^(gamma/(gamma-1));

P_t9_9_values(i) = pi_c * pi_r_values(i) * pi_t_values(i);

M_9_values(i) = sqrt(2/(gamma-1)*(P_t9_9_values(i)^((gamma-1)/gamma)-1));

T_9_T_0_values(i) = t_lambda_values(i) * t_t_values(i) / (P_t9_9_values(i)^((gamma-1)/gamma));

V_9_a_0_values(i) = M_9_values(i)*sqrt(T_9_T_0_values(i));

thrust_values(j,i) = a_0_values(i)* (V_9_a_0_values(i)- M_0);

TSFC_values(j,i) = f_values(i)/thrust_values(j,i);

eff_p_values(j,i) = 2 * V_0_values(i) * thrust_values(j,i) / (a_0_values(i)^2 * (V_9_a_0_values(i)^2 - M_0^2));

eff_th_values(j,i) = (a_0_values(i)^2 * (V_9_a_0_values(i)^2 - M_0^2)) / (2 * f_values(i) * Qr);

eff_o_values(j,i)  = eff_p_values(j,i) * eff_th_values(j,i);

end

end

figure;

plot(H_values,thrust_values(1,:),'r','LineWidth',2);

title('Variation of Thrust with Height');

xlabel('Height');

ylabel('Thrust');

hold on

plot(H_values,thrust_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,TSFC_values(1,:),'r','LineWidth',2);

title('Variation of TSFC with Height');

xlabel('Height');

ylabel('TSFC');

hold on

plot(H_values,TSFC_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_p_values(1,:),'r','LineWidth',2);

title('Variation of efficiency of propulsion with Height');

xlabel('Height');

ylabel('efficiency of propulsion');

hold on

plot(H_values,eff_p_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_th_values(1,:),'r','LineWidth',2);

title('Variation of efficiency of thermal energy with Height');

xlabel('Height');

ylabel('efficiency of th');

hold on

plot(H_values,eff_th_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_o_values(1,:),'r','LineWidth',2);

title('Variation of total efficiency with Height');

xlabel('Height');

ylabel('total efficiency');

hold on

plot(H_values,eff_o_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

Part(b)

clc;

clear;

clear all;

%known parameters

H_values = linspace(0,15000,1000);%%define the height

M_0_values = [0.85 2.1];

gamma = 1.4;

R = 287;

C_p = R * gamma/(gamma-1);

T_t4 = 1900;

Qr = 43000000;

%need to be known

T_0_values = zeros(size(H_values));

a_0_values = zeros(size(H_values));

V_0_values = zeros(size(H_values));

t_r_values = zeros(size(H_values));

t_lambda_values = zeros(size(H_values));

pi_r_values = zeros(size(H_values));

t_c_values = zeros(size(H_values));

pi_c_values = zeros(length(M_0_values),length(H_values));

f_values = zeros(size(H_values));

t_t_values = zeros(size(H_values));

pi_t_values = zeros(size(H_values));

P_t9_9_values = zeros(size(H_values));

M_9_values = zeros(size(H_values));

T_9_T_0_values = zeros(size(H_values));

V_9_a_0_values = zeros(size(H_values));

thrust_values = zeros(length(M_0_values),length(H_values));

TSFC_values = zeros(length(M_0_values),length(H_values));

eff_p_values = zeros(length(M_0_values),length(H_values));

eff_th_values = zeros(length(M_0_values),length(H_values));

eff_o_values = zeros(length(M_0_values),length(H_values));

%definr the T_0

for j = 1:length(M_0_values)  % 遍历不同的 M_0

M_0 = M_0_values(j);  % 当前 M_0

for i = 1:length(H_values)

H = H_values(i);

if H < 11000

T_0_values(i) = 288.15-0.0065*H;

else

T_0_values(i) = 216.65;

end

a_0_values(i) = sqrt(gamma * R .* T_0_values(i));

V_0_values(i) = a_0_values(i) * M_0;

t_r_values(i) = 1+(gamma-1)/2 * M_0^2;

pi_r_values(i) = t_r_values(i)^(gamma/(gamma - 1));

t_lambda_values(i) = T_t4 / T_0_values(i);

t_c_values(i) = sqrt(t_lambda_values(i)) / t_r_values(i);

pi_c_values(j,i) = t_c_values(i)^(gamma/(gamma-1));

f_values(i) = (t_lambda_values(i)-t_r_values(i) * t_c_values(i)) ./ (Qr/(C_p * T_0_values(i))-t_lambda_values(i));

t_t_values(i) = 1-t_r_values(i)*(t_c_values(i)-1)/t_lambda_values(i);

pi_t_values(i) = t_t_values(i)^(gamma/(gamma-1));

P_t9_9_values(i) = pi_c_values(j,i) * pi_r_values(i) * pi_t_values(i);

M_9_values(i) = sqrt(2/(gamma-1)*(P_t9_9_values(i)^((gamma-1)/gamma)-1));

T_9_T_0_values(i) = t_lambda_values(i) * t_t_values(i) / (P_t9_9_values(i)^((gamma-1)/gamma));

V_9_a_0_values(i) = M_9_values(i)*sqrt(T_9_T_0_values(i));

thrust_values(j,i) = a_0_values(i)* (V_9_a_0_values(i)- M_0);

TSFC_values(j,i) = f_values(i)/thrust_values(j,i);

eff_p_values(j,i) = 2 * V_0_values(i) * thrust_values(j,i) / (a_0_values(i)^2 * (V_9_a_0_values(i)^2 - M_0^2));

eff_th_values(j,i) = (a_0_values(i)^2 * (V_9_a_0_values(i)^2 - M_0^2)) / (2 * f_values(i) * Qr);

eff_o_values(j,i)  = eff_p_values(j,i) * eff_th_values(j,i);

end

end

figure;

plot(H_values, pi_c_values(1,:), 'b', 'LineWidth', 2);

hold on;

title('Optimal \pi_c for Maximum Thrust vs Altitude');

xlabel('Altitude (m)');

ylabel('Optimal \pi_c');

plot(H_values, pi_c_values(2,:), 'r', 'LineWidth', 2);

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

figure;

plot(H_values,thrust_values(1,:),'r','LineWidth',2);

title('Variation of Thrust with Height');

xlabel('Height');

ylabel('Thrust');

hold on

plot(H_values,thrust_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,TSFC_values(1,:),'r','LineWidth',2);

title('Variation of TSFC with Height');

xlabel('Height');

ylabel('TSFC');

hold on

plot(H_values,TSFC_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_p_values(1,:),'r','LineWidth',2);

title('Variation of efficiency of propulsion with Height');

xlabel('Height');

ylabel('efficiency of propulsion');

hold on

plot(H_values,eff_p_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_th_values(1,:),'r','LineWidth',2);

title('Variation of efficiency of thermal energy with Height');

xlabel('Height');

ylabel('/eta_t_h');

hold on

plot(H_values,eff_th_values(2,:),'b','LineWidth',1.5);

legend('M=0.85','M=2.1');

grid on;

figure;

plot(H_values,eff_o_values(1,:),'r','LineWidth',2);

title('Variation of total efficiency with Height');

xlabel('Height');

ylabel('total efficiency');

hold on

plot(H_values,eff_o_values(2,:),'b','LineWidth',2);

legend('M=0.85','M=2.1');

grid on;

Part(c)

%%

clc;

clear;

clear all;

%known parameters

H_values = linspace(0,15000,1000);%%define the height

M_0_values = [0.85 2.1];

T_t4 = 1900;

pi_c = 15;

Qr = 43000000;

pi_d = 0.98;

e_c = 0.85;

pi_b = 0.95;

eta_b = 0.98;

e_t = 0.85;

eta_m = 0.98;

pi_n = 0.97;

gamma_c = 1.4;

C_pc = 1004;

gamma_t = 1.33;

C_pt = 1156;

R_c = (gamma_c-1)/gamma_c * C_pc;

R_t = (gamma_t-1)/gamma_t * C_pt;

%need to be known

T_0_values = zeros(size(H_values));

a_0_values = zeros(size(H_values));

V_0_values = zeros(size(H_values));

t_r_values = zeros(size(H_values));

t_lambda_values = zeros(size(H_values));

pi_r_values = zeros(size(H_values));

t_c_values = zeros(size(H_values));

eff_c_values = zeros(size(H_values));

f_values = zeros(size(H_values));

t_t_values = zeros(size(H_values));

pi_t_values = zeros(size(H_values));

eta_t_values = zeros(size(H_values));

P_t9_9_values = zeros(size(H_values));

M_9_values = zeros(size(H_values));

T_9_T_0_values = zeros(size(H_values));

V_9_a_0_values = zeros(size(H_values));

thrust_values = zeros(size(H_values));

TSFC_values = zeros(size(H_values));

eff_p_values = zeros(size(H_values));

eff_th_values = zeros(size(H_values));

eff_o_values = zeros(size(H_values));

%definr the T_0

for j = 1:length(M_0_values)  % different M_0

M_0 = M_0_values(j);

for i = 1:length(H_values)

H = H_values(i);

if H < 11000

T_0_values(i) = 288.15-0.0065*H;

else

T_0_values(i) = 216.65;

end

a_0_values(i) = sqrt(gamma_c * R_c * T_0_values(i));

V_0_values(i) = a_0_values(i) * M_0;

t_r_values(i) = 1+(gamma_c-1)/2 * M_0^2;

pi_r_values(i) = t_r_values(i)^(gamma_c/(gamma_c - 1));

t_lambda_values(i) = T_t4 * C_pt / (T_0_values(i) * C_pc);

t_c_values(i) = pi_c^((gamma_c - 1)/(gamma_c * e_c));

eff_c_values(i) = (pi_c^((gamma_c - 1)/gamma_c)-1)/(t_c_values(i) - 1);

f_values(i) = (t_lambda_values(i)-t_r_values(i) * t_c_values(i)) / ((eta_b * Qr)/(C_pc * T_0_values(i))-t_lambda_values(i));

t_t_values(i) = 1 - (t_r_values(i)*(t_c_values(i)-1)/t_lambda_values(i))/(eta_m * (1 + f_values(i)));

pi_t_values(i) = t_t_values(i)^(gamma_t /((gamma_t-1)*e_t));

eta_t_values(i) = (1-t_t_values(i)) / (1 - t_t_values(i)^(-e_t));

P_t9_9_values(i) = pi_c * pi_r_values(i) * pi_t_values(i) * pi_n * pi_b * pi_d;

M_9_values(i) = sqrt(2/(gamma_t - 1)*(P_t9_9_values(i)^((gamma_t - 1)/gamma_t)-1));

T_9_T_0_values(i) = t_lambda_values(i) * t_t_values(i) / P_t9_9_values(i)^((gamma_t-1)/gamma_t)*C_pc/C_pt;

V_9_a_0_values(i) = M_9_values(i)*sqrt(T_9_T_0_values(i) * R_t/R_c * gamma_t/gamma_c);

thrust_values(j,i) = a_0_values(i)* ((1 + f_values(i)) * V_9_a_0_values(i)- M_0);

TSFC_values(j,i) = f_values(i)/thrust_values(j,i);

eff_p_values(j,i) = 2 * V_0_values(i) * thrust_values(j,i) / (a_0_values(i)^2 * ((1 + f_values(i)) * V_9_a_0_values(i)^2 - M_0^2));

eff_th_values(j,i) = a_0_values(i)^2 * ((1 + f_values(i)) * (V_9_a_0_values(i)^2 - M_0^2)) / (2 * f_values(i) * Qr);

eff_o_values(j,i)  = eff_p_values(j,i) * eff_th_values(j,i);

end

end

figure;

plot(H_values, thrust_values(1,:), 'r', 'LineWidth', 2);

hold on;

plot(H_values, thrust_values(2,:), 'b', 'LineWidth', 2);

title('Thrust vs Altitude');

xlabel('Altitude (m)');

ylabel('Specific Thrust');

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

figure;

plot(H_values, TSFC_values(1,:), 'r', 'LineWidth', 2);

hold on;

plot(H_values, TSFC_values(2,:), 'b', 'LineWidth', 2);

title('TSFC vs Altitude');

xlabel('Altitude (m)');

ylabel('TSFC');

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

figure;

plot(H_values, eff_p_values(1,:), 'r', 'LineWidth', 2);

hold on;

plot(H_values, eff_p_values(2,:), 'b', 'LineWidth', 2);

title('\eta_p vs Altitude');

xlabel('Altitude (m)');

ylabel('\eta_p');

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

figure;

plot(H_values, eff_th_values(1,:), 'r', 'LineWidth', 2);

hold on;

plot(H_values, eff_th_values(2,:), 'b', 'LineWidth', 2);

title('\eta_t_h vs Altitude');

xlabel('Altitude (m)');

ylabel('\eta_t_h');

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

figure;

plot(H_values, eff_o_values(1,:), 'r', 'LineWidth', 2);

hold on;

plot(H_values, eff_o_values(2,:), 'b', 'LineWidth', 2);

title('\eta_o vs Altitude');

xlabel('Altitude (m)');

ylabel('\eta_o');

legend('M_0 = 0.85', 'M_0 = 2.1');

grid on;

hold off

part (d)

%%

clc;

clear;

clear all;

% 已知参数

H_values = linspace(0, 15000, 1000);  % 定义高度范围

M_0_values = [0.85, 2.1];  % 飞行马赫数

T_t4 = 1900;

pi_c = 15;

Qr = 43000000;

pi_d = 0.98;

e_c = 0.85;  % e_c 取10个值

pi_b = 0.95;

eta_b = 0.98;

e_t_values = linspace(0.833, 0.867, 10);

eta_m = 0.98;

pi_n = 0.97;

gamma_c = 1.4;

C_pc = 1004;

gamma_t = 1.33;

C_pt = 1156;

R_c = (gamma_c - 1) / gamma_c * C_pc;

R_t = (gamma_t - 1) / gamma_t * C_pt;

% 初始化存储数组

thrust_values = zeros(length(e_t_values), length(H_values), length(M_0_values));

% 遍历 M_0

figure;

hold on;

colors = lines(10);  % 生成 10 种不同颜色

for k = 1:length(M_0_values)

M_0 = M_0_values(k);

% 遍历 e_c

for j = 1:length(e_t_values)

e_t = e_t_values(j);

% 遍历高度 H

for i = 1:length(H_values)

H = H_values(i);

% 计算环境温度

if H < 11000

T_0 = 288.15 - 0.0065 * H;

else

T_0 = 216.65;

end

% 计算气动力参数

a_0 = sqrt(gamma_c * R_c * T_0);

V_0 = a_0 * M_0;

t_r = 1 + (gamma_c - 1) / 2 * M_0^2;

pi_r = t_r^(gamma_c / (gamma_c - 1));

t_lambda = T_t4 * C_pt / (T_0 * C_pc);

t_c = pi_c^((gamma_c - 1) / (gamma_c * e_c));

eff_c = (pi_c^((gamma_c - 1) / gamma_c) - 1) / (t_c - 1);

f = (t_lambda - t_r * t_c) / ((eta_b * Qr) / (C_pc * T_0) - t_lambda);

t_t = 1 - (t_r * (t_c - 1) / t_lambda) / (eta_m * (1 + f));

pi_t = t_t^(gamma_t / ((gamma_t - 1) * e_t));

eta_t = (1 - t_t) / (1 - t_t^(-e_t));

P_t9_9 = pi_c * pi_r * pi_t * pi_n * pi_b * pi_d;

M_9 = sqrt(2 / (gamma_t - 1) * (P_t9_9^((gamma_t - 1) / gamma_t) - 1));

T_9_T_0 = t_lambda * t_t / P_t9_9^((gamma_t - 1) / gamma_t) * C_pc / C_pt;

V_9_a_0 = M_9 * sqrt(T_9_T_0 * R_t / R_c * gamma_t / gamma_c);

% 计算推力

thrust_values(j, i, k) = a_0 * ((1 + f) * V_9_a_0 - M_0);

end

% 画出推力随高度变化的曲线

if k == 1

plot(H_values, squeeze(thrust_values(j, :, k)), 'Color', colors(j, :), 'LineWidth', 1.5);

else

plot(H_values, squeeze(thrust_values(j, :, k)), '--', 'Color', colors(j, :), 'LineWidth', 1.5);

end

end

end

% 设置图例、标签和标题

title('Thrust vs Altitude for Different e_t and M_0');

xlabel('Altitude (m)');

ylabel('Thrust (N)');

legend_labels = cell(1, 20);

for j = 1:10

legend_labels{j} = sprintf('M_0=0.85, e_t=%.3f', e_t_values(j));

legend_labels{j + 10} = sprintf('M_0=2.1, e_t=%.3f', e_t_values(j));

end

legend(legend_labels, 'Location', 'northeastoutside');

grid on;

hold off;

part(e)

%%

clc;

clear;

clear all;

% Given parameters

H = 10000; % Altitude in meters

M_0_values = [0.85, 2.1]; % Mach numbers

T_t4 = 1900; % Turbine inlet temperature in Kelvin

pi_c_values = linspace(1, 100, 100); % Range of compression ratios

Qr = 43000000; % Fuel heating value in J/kg

pi_d = 0.98;

e_c = 0.85;

pi_b = 0.95;

eta_b = 0.98;

e_t = 0.85;

eta_m = 0.98;

pi_n = 0.97;

gamma_c = 1.4;

C_pc = 1004;

gamma_t = 1.33;

C_pt = 1156;

R_c = (gamma_c - 1) / gamma_c * C_pc;

R_t = (gamma_t - 1) / gamma_t * C_pt;

% Calculate ambient temperature T_0

if H < 11000

T_0 = 288.15 - 0.0065 * H;

else

T_0 = 216.65;

end

% Preallocate arrays for thrust

thrust_values = zeros(length(M_0_values), length(pi_c_values));

% Create figure

figure;

hold on;

colors = lines(length(M_0_values)); % Generate different colors for the curves

for j = 1:length(M_0_values)

M_0 = M_0_values(j);

% Compute initial parameters

a_0 = sqrt(gamma_c * R_c * T_0);

V_0 = a_0 * M_0;

t_r = 1 + (gamma_c - 1) / 2 * M_0^2;

pi_r = t_r^(gamma_c / (gamma_c - 1));

for i = 1:length(pi_c_values)

pi_c = pi_c_values(i);

t_lambda = T_t4 * C_pt / (T_0 * C_pc);

t_c = pi_c^((gamma_c - 1) / (gamma_c * e_c));

eff_c = (pi_c^((gamma_c - 1) / gamma_c) - 1) / (t_c - 1);

f = (t_lambda - t_r * t_c) / (((eta_b * Qr) / (C_pc * T_0)) - t_lambda);

t_t = 1 - (t_r * (t_c - 1) / t_lambda) / (eta_m * (1 + f));

pi_t = t_t^(gamma_t / ((gamma_t - 1) * e_t));

eta_t = (1 - t_t) / (1 - t_t^(-e_t));

P_t9_9 = pi_c * pi_r * pi_t * pi_n * pi_b * pi_d;

M_9 = sqrt(2 / (gamma_t - 1) * (P_t9_9^((gamma_t - 1) / gamma_t) - 1));

T_9_T_0 = t_lambda * t_t / P_t9_9^((gamma_t - 1) / gamma_t) * C_pc / C_pt;

V_9_a_0 = M_9 * sqrt(T_9_T_0 * R_t / R_c * gamma_t / gamma_c);

% Compute thrust

thrust_values(j, i) = a_0 * ((1 + f) * V_9_a_0 - M_0);

end

% Plot thrust vs pi_c

plot(pi_c_values, thrust_values(j, :), 'Color', colors(j, :), 'LineWidth', 1.5, ...

'DisplayName', sprintf('M_0 = %.2f', M_0));

% Find and mark the maximum thrust point

[max_thrust, max_idx] = max(thrust_values(j, :));

best_pi_c = pi_c_values(max_idx);

% Highlight the maximum thrust point

plot(best_pi_c, max_thrust, 'o', 'Color', colors(j, :), 'MarkerSize', 8, 'MarkerFaceColor', colors(j, :));

text(best_pi_c, max_thrust, sprintf('\\leftarrow Max Thrust: %.1f N (\\pi_c = %.1f)', max_thrust, best_pi_c), ...

'Color', colors(j, :), 'FontSize', 10);

end

% Labels and title in English

xlabel('\pi_c (Compression Ratio)', 'FontSize', 12);

ylabel('Thrust (N)', 'FontSize', 12);

title('Thrust vs Compression Ratio for Different Mach Numbers', 'FontSize', 14);

legend('Location', 'best');

grid on;

hold off;